HOW TO INSTALL DHCP-SERVER

- Got to the Windows Network Adapters

- Choose the network card you want to use(additional network card is recommended)

- Set a fixed ip adresse :

IP: 	169.254.0.1 (z.B.)
Mask: 	255.255.255.0

- Unzip DCHP-Server tool to your favorized location

- Edit  dhcpsrv.ini:

Change both paths to match the folder you have unzipped dhcp-server

- Install DHCP

-start ‘dhcpsrv.exe ‘

-1.change to admin mode

-2.configure windows firewall rule

-3.install dhcp as windows service

-4.start service

- You are now done with the installation.

When connection a controller you now should see the mac adresse and the according ip adress set by the dhcp-server at :

http://127.0.0.1/dhcpstatus.xml